Rio de Truchas Line

In 2017 SAGE acquired a 6.1 km seismic line along the Rio de Truchas, in Rio Arriba County, NM.  Rio de Truchas is a seasonally dry, southeast-to-northwest trending riverbed that originates in the Sangre de Cristo mountains – below Truchas peak – and terminates in the Rio Grande river between Lyden, NM and Velarde, NM, which is approximately 52 km north of downtown Santa Fe, NM.  The first source point of the line (100) was located within the Rio de Truchas approximately 1km southeast of the intersection of the riverbed with State Road 68.  The last source point on the line (301) was located within the Rio de Truchas approximately 6km southeast of the intersection of the riverbed with State Road 68.

Acquisition

Source

Type					Vibroseis

Make

Ground Force

Sweep Frequencies			4-80Hz

Sweep Length				12000 ms

Start Taper				500 ms

End Taper				500 ms

Taper Type				Cos Squared

Number of Sweeps / Vibe Point	3

Number of Vibes / Source Pattern	1

Receivers

Type					3 Component

Elements				SM-7/U-B 10Hz

Number of geophones per group	1

Spread

Number of channels			360

Number of stations			120

Pattern 				Shot through fixed spread.

Source Interval			20m, on the half-station.

Receiver Interval			20m

Recording

Type					Aries XP

Data Format				SEGY Revision 0

Sample Format			IEEE Floating Point. Little Endian

Sample Interval			2 ms

Samples per Trace			2501

Record Length				5000 ms

Traces per record			363

Auxiliaries per record			3 (Channels 361, 362, 363)

Low Cut Filter				3 Hz

High Cut Filter				205 Hz

Stack of Rio de Truchas line.   Display is from West to East.

Archive

Raw SEGY field files – 4 second records

00000001.SGY

00000002.SGY

00000003.SGY

00000004.SGY

.

.

.

00000252.SGY

Raw SEGY field files – 4 second records

00000253.SGY

00000254.SGY

00000255.SGY

00000256.SGY

.

.

.

00000789.SGY

Shell Processing Support (SPS) data

2017.sps	- 	Source Point Record

2017.rps	- 	Receiver Point Record

2017.xps	- 	Relational Record

SEGY file with source and receiver geometry in trace headers

Rio de Trucas - geometry.sgy