Camino del Cerezo Line

In 2018 SAGE acquired a 6.1 km seismic line along the Camino del Cerezo, in Santa Fe County, NM.  Camino del Cerezo is a dirt road that parallels a seasonally dry, east-to-west trending riverbed northwest of Santa Fe, NM.  Camino del Cerezo intersects Old Buckman Road (Hwy 77) approximately 1km south of the riverbed and 19km northwest of downtown Santa Fe.  The first source point of the line (100) was located on Camino del Cerezo approximately 1km east of Old Buckman Road.  The last source point on the line (301) was located on the Camino del Cerezo approximately 6km east of Old Buckman Road.

Acquisition

Source

Type					Vibroseis

Make

Ground Force

Sweep Frequencies			4-60Hz

Sweep Length				10000 ms

Start Taper				500 ms

End Taper				500 ms

Taper Type				Cos Squared

Number of Sweeps / Vibe Point	3

Number of Vibes / Source Pattern	1

Receivers

Type					3 Component

Elements				SM-7/U-B 10Hz

Number of geophones per group	1

Spread

Number of channels			360

Number of stations			120

Pattern 				Shot through fixed spread.

Source Interval			20m, on the half-station.

Receiver Interval			20m

Recording

Type					Aries XP

Data Format				SEGY Revision 0

Sample Format			IEEE Floating Point. Little Endian

Sample Interval			2 ms

Samples per Trace			2501

Record Length				5000 ms

Traces per record			363

Auxiliaries per record			3 (Channels 361, 362, 363)

Low Cut Filter				3 Hz

High Cut Filter				205 Hz

Stack of Camino del Cerezo Line.   Display is from West to East.

Archive

Raw SEGY field files

00000001.SGY

00000002.SGY

00000003.SGY

00000004.SGY

.

.

.

00000756.SGY

Shell Processing Support (SPS) data

2018.sps	- 	Source Point Record

2018.rps	- 	Receiver Point Record

2018.xps	- 	Relational Record

SEGY file with source and receiver geometry in trace headers

Camino Cerezo - geometry.sgy